Exam : Model answer

Exercise 1 : ( 7 Points)

Consider the circuit shown in the figure below :

R2

- Function of the circuit : Vs=- R2R1Ve ;

Inverting amplifier

R2

Ve

Vs

-

+

R1

Model of the noisy assembly:

R2

R1

Ve

Vs

Types of internal noise:  Thermal noise

Shot noise

G1=-3dB

F1=3dB

G2=14dB

F2=5dB

G3=20dB

F3=10dB

Block diagram of the receiving chain :

- Friiss theorem : F= F1+ F2-1A1+ F3-1A1A2        F(dB) = 10 log F   ;    G(dB) = 10 log A

F1 = 3dB ;   F1 = 103/10 = 1,99

G1 = -3dB (pertes) ; A1 = 10-3/10 = 0,5

F2 = 5dB ;   F2 = 105/10 = 3,16                   F= F1+ F2-1A1+ F3-1A1A2 = 1,99+ 3,16-10,5+ 10-10,5x25,12=7,03

G2 = -3dB ; A2 = 1014/10 = 25,12

F3 = 10dB ;   F3 = 1010/10 = 10

G3 = 20dB ;  A3 = 1020/10 = 100

- By swapping the first and second elements of the chain, calculate the noise factor in this case

F= F2+ F1-1A2+ F3-1A1A2 = 3,16+ 1,99-125,12+ 10-10,5x25,12= 3,92

- Conclusion : The noise factor in the second case is lower; the noise at the input of the receiving chain is reduced.

Exercise 2 : ( 6 Points)

The DC voltage E is equivalent to a very large (infinite) capacitance, closed circuit.

It does not exhibit any capacitive effect because the frequency is zero

R1

R2

V1(t)

C12

1. Capacitance resulting from capacitive coupling                                2. Equivalent electric circuit

V1

V2

C1

C12

V1

R1

R1	                             R2

C1

R2                V2

3. Expression for the voltage ratio  V2V1

En appliquant la règle du diviseur de tension on a :

V2= R2R2+ 1jωC12V1= R21+jωR2C12jωC12V1= jR2C12ω1 + jR2C12ω V1

V2V1= jR2C12ω1 + jR2C12ω

4. Study of  the low and high frequency cases. Conclusion

- LF (f → 0) : V2V1=0 →  V2=0: the cable is not disrupted at LF,

- HF (f →∞) : V2V1 = 1 → V2 = V1 : the cable is disrupted at HF.

Exercise 3 : ( 7 Points)

I.

1.  Schematic diagram

I1                                       I

2. Coupling type: Low-frequency (LF) conduction coupling.

The load is the source that generates the disturbance, and the network is the target that receives it.3.

- Coupling mode : Differential mode

4. Protection technique: Tuned passive filter

5. Protection is required for the second type of load (lighting) because the current drawn by this load contains the 3rd,

5th and 7th harmonics. The first type of load (resistive load) does not require protection; the current drawn in this case contains only the fundamental frequency.

II.

1. State the frequency values for the various currents and spectral representation

I1 : fondamental          f0 = 50Hz

I3 : Harmonic 3            f3 = 3f0 = 150Hz

I5 : Harmonic 5            f5 = 5f0 = 250Hz

I7 : Harmonic 7            f7 = 7f0 = 350Hz

2.

I1             I1+I3+ I5+I7

I7

220 V

50 Hz

Electric diagram

3. Protection technique : Tuned passive filter

4. Value of C3 for harmonic 3.

For L = 1mH    f3 = 12πLC3=150 Hz   f32= 14π2LC3    soit C3=14π2f32L = 1,13 mF